1）: Warning

1:For the first 50 hours, please don’t let it running at full capacity.

2: Please change engine oil after 50 hours.

3: Don’t let it running in low or high speed for long time, keep normal please.

4: Keep it with no load running for 3-5 minutes.

5:If there is something wrong with it, stop it and eliminate it. Don’t keep it working with troubles.

Don’t keep it running with no body around.

6: Stop it normally.

7: Make sure water for water tank is clean.

8: Use the right and clean engine oil and diesel oil, change engine oil regularly.

9: Fill engine oil in ail filter and clean it and change engine oil regularly.

10:Prohibit water in the intake and exhaust value.

11: When the tempreture is ＜0°C, get out the cooling water.

12:Don’t let child, the old be around when it is running.

13: Don’t use it when drunk.

14:Keep inflammable far away from the engine.

15:Make sure stop it before fill diesel oil.

16: Make sure it is adequate ventilation of the yard when it is running.

17: Make sure stop it before overhaul.

18: Don’t touch high temperature parts when it is running or just stops.

19: Pay more attention when get the engine oil out; get the cooling water out when the engine is cooled down.

20: Hold tidely the hand shank when start it.

2）: engine oil

1:Use CC-30 or CC-40 engine oil when local temperature is ≥0°C, use CC10W/30 when local temperature is ＜0°C.

2：Get the scale out and fill clearn engine oil around 2.5 kg, check the oil level between the 2 lines.

(Photos for view.)

3: Don’t fill too much over the top line, when it’s lower the bottom line, please fill it, keep the oil level between the 2 lines.

3）: Diesel oil

1: Use #0 when local temperature is ≥0°C, use #-10 or -20 when local temperature is ＜0°C.

2: fill clean diesel oil like the photo 3

3: Get the air out from the tube.(it it ok when no bubbles of flowing diesel oil) . photo 4.

photo 3                                           photo 4

4）: cooling water

Use clean soft water as cooling water, fill the water until the floater can not going up.

5): diesel working

1: after starting, please check the red mark of engine oil indicator valve if going up, if not, please

stop the machine and fill more engine oil or eliminate the defect.

2: when it is working, the cooling water should be in boiling state. And please add water timely.

3:when there is black smoking, please make it lesser loaded or stop it for defect elimination.

4:If there is strange noise of the machine,please stop it and eliminate the defect.

5:After first 50 hr, please check and tiden all the blots and nuts.

6): stopping of diesel

1: Before stop it, keep it running serval minutes with no loaded.

2: When emergency stop needed, block the inlet tube of air filter with towel. (see photo please)

7): tips of after stopping of diesel

1:turn off the oil tank.

2: for a long time no working or in winter, get out the cooling water to put it away.

3: Clean the water chamber regularly.

4: Close the inlet and exhaust valve (photo).

8): maintenance

1: adding cooling water according the above mentioned situation.

2: adding engine oil every day (according the above mentioned situation).

3:.oil strainer: after the first 50 hours, change new engine oil and clean the oil filter

Afterwhile, change new engine oil and clean the oil filter every 100hr.

4: air filter: after the first 50 hours, change new engine oil and clean the oil filter

Afterwhile, change new engine oil and clean the oil filter every 100hr.

5:diesel filter: clean the core with diesel oil, when the core is broken, change it timely.

6: nut of cylinder cover: after the first 30 hrs, check and make it tight.

7: oil tank: clean it with clean diesel every 50hr.

8: adjust the valve clearance every 100 hr.